# Three Replacement Bridges — Design Development

P4 | 25 October 2026 | Construction release deferred

This package develops the supplied structural criteria and replacement studies into a traceable preliminary engineering basis, calculation sheets and source-based detailing. It does not constitute completed member, connection, bearing, fatigue or marine-operation design. Reported structural analysis results remain attributed to the source studies.

# 1. Scope and source hierarchy

Scope: ICW–ICG LD, ICP–ICG LD and BHS–SLQ replacement/new bridges, with landings, platform/jacket interfaces, access fittings and service supports. BHS includes service-transfer and demolition interfaces. BRRP-specific PartII is the project basis; other-project PartII content in Section3 is not imported.

Principal sources: Design Criteria & Execution Methodology.pdf (PartI Rev14 pp1–116; BRRP PartII pp117–120; PartIII pp121–132; methodology RevC pp133–154); SOW & Bridge Report Combined.pdf (structural scope pp15–53 and reports as below); MTO Drawings IRT.pdf; structural addendum; Specification Combined.pdf. Source IDs, revisions and applicable clauses govern; conflicts remain queries.

| Bridge | Study L / approximate gap | Sliding / fixed intent | Study report / combined PDF pages |
| --- | --- | --- | --- |
| ICW-ICG LD | 56 / 54.5 m | ICW / ICG | BPR-0004 Rev0 / 133-135;152-153 |
| ICP-ICG LD | 84 / 80.5 m | ICP / ICG | BPR-0003 Rev0 / 196-199;207-209 |
| BHS-SLQ LD | 42 / 40 m | BHS / SLQ | BPR-0005 RevA / 97-100;122-123 |

# 2. Consolidated design basis

Design life: 25 years. Source: BRRP PartII p119. CONFIRMED.

Elastic modulus E: 210000 MPa. Source: PartI Annexure2 p66. CONFIRMED.

Shear modulus G: 80770 MPa. Source: PartI Annexure2 p66. CONFIRMED.

Poisson ratio: 0.3 ratio. Source: PartI Annexure2 p66. CONFIRMED.

Steel density: 7850 kg/m3. Source: PartI Annexure2 p66. CONFIRMED.

Gravity: 9.81 m/s2. Source: PartI Annexure2 p66. CONFIRMED.

Thermal expansion: 1.2e-05 per C. Source: PartI Annexure2 p66. CONFIRMED.

Global walkway loading: 250 kg/m2. Source: Bridge clause p42; Annexure4. CONFIRMED.

Local grating loading: 500 kg/m2. Source: Annexure4 p70. CONFIRMED.

Minimum clear walkway: 1.2 m. Source: Bridge clause p43. CONFIRMED.

Bridge vertical limit: L/400 mm consistent with L. Source: p43;Annexure12 pp85-86. CONFIRMED.

Bridge lateral wind limit: L/500 mm consistent with L. Source: Annexure12 p86. CONFIRMED.

Differential translation multiplier: 1.5 ratio. Source: Bridge clause p43. CONFIRMED; input movements pending.

Hinged axial thrust multiplier: 1.5 ratio. Source: Bridge clause p43. CONFIRMED; thrust envelope pending.

Future piping operating-load allowance: 20 percent. Source: Bridge clause p42. CONFIRMED; included/excluded basis to reconcile.

Camber: Compensate dead-load deflection profile. Source: Bridge clause p43. RULE CONFIRMED; magnitude/profile pending.

Wind gust for56m/84m: 15 seconds. Source: Annexure5 p73. CONFIRMED.

Wind gust for42m: 3 seconds. Source: Annexure5 p73. CONFIRMED.

Environment: MH North and South field. Source: PartII p119;Annexure13A. CONFIRMED; height adjustment/calculation pending.

Material procurement: Spec6001F Rev14 requirements. Source: Specification Combined pp169-243. Grade/toughness selection pending.

Welding/NDT: FS2009F Rev8 requirements. Source: Specification Combined pp3-38. Apply by joint and inspection plan.

Coating: FS2005 Rev9 requirements. Source: Specification Combined pp91-168. Exposure-specific system selection pending.

Roof: Process-platform connecting bridges; slope minimum1:50 applicability. Source: Bridge clause p43. Confirm bridge-specific implementation.

Location tolerance: +1.0m as written in source requirement. Source: Bridge clause p43. Clarify interpretation for existing supports; NOT bearing travel.

# 3. Load model preparation

Separate structural self-weight, non-generated dead loads, services, walkway loading, local grating loading, operating contents and future allowances. Record source revision, included contingencies and area/length distribution. Source report totals are not directly transferred to a revised geometry without reconciliation.

| Bridge | Report DL | NGDL | Cable | Walkway LL | Piping operating |
| --- | --- | --- | --- | --- | --- |
| ICW-ICG LD | 418.03 | 155.75 | 0.75 | 156.25 | 0 |
| ICP-ICG LD | 966.64 | 156.73 | 0.882 | 288 | 0 |
| BHS-SLQ LD | 789.79 | 266.47 | 77.66 | 268.42 | 1440 |

Table units kN; transcribed from source load tables. Source tables and generated output differ in some entries; these are not final design totals.

LC01 In-place extreme: Bridge dead + services dead + piping operating contents + extreme wind + friction. Include applicable allowances; verify source load map.

LC02 In-place operating: Bridge dead + services dead + piping contents + walkway LL + monorail LL + operating wind + friction. Assess both wind signs/directions and service configurations.

LC03 Dead / dry reference: Bridge dead + piping/cable/tray dead. Camber and construction-setting reference must be defined.

LC04 Hydrotest: Bridge dead + services dead + one large pipe hydrotest at a time. Check actual line/test sequence; not all lines assumed simultaneous.

LC05 Movement / supports: Relative platform translations and compatible rotations. 150% specified translation;150% expected hinged axial thrust; no double counting.

LC06 VIV / fatigue: Wind-induced vibration of truss members. Displacement and fatigue damage; in-service and applicable transport states.

LC07 Fabrication / weighing: Actual support points and construction loads. Check changed support configurations and local effects.

LC08 Loadout: Actual loadout support positions / loss of support / yard bearing. Procedure-specific; applicable three-point cases.

LC09 Transport: Actual barge/vessel motions, cargo mass/COG and restraints. Criteria preliminary motions are screening basis; final route/spread analysis required.

LC10 Lifting / landing: Rigging geometry, skew distribution, weight/COG and dynamics. Operation-specific 3D model; no approved lift points defined.

LC11 Local grating / fittings: Local element loading and appropriate concentrated loads. 500kg/m2 grating local check distinct from250kg/m2 global walkway.

LC12 Landings / jackets: Concurrent signed bridge reactions plus existing platform loads. Incorporate addendum strengthening and actual condition.

BHS future-piping sensitivity: if the reported1440kN represents the same operating-load basis before the20% future allowance, the corresponding value is1728kN. Do not apply this increment until existing allowance treatment is confirmed. Operating contents, empty pipe weight and local support actions must remain consistent.

# 4. New arithmetic: walkway and local grating loads

Using criteria g=9.81m/s2: global walkway q=250x9.81/1000=2.4525kN/m2; local grating q=500x9.81/1000=4.905kN/m2. Line load is q times actual loaded width. Rectangle load is q x studyL x report loading width; this is a reconciliation screen, not a substitute for the real load map.

| Bridge | Line load kN/m | Rectangle load kN | Source walkway LL kN |
| --- | --- | --- | --- |
| ICW-ICG LD | 2.9430 | 164.808 | 156.25 |
| ICP-ICG LD | 2.9430 | 247.212 | 288 |
| BHS-SLQ LD | 5.6996 | 239.384 | 268.42 |

ICW source walkway table156.25kN differs from output144.69kN. ICP source288kN and BHS268.42kN do not equal the simple rectangles shown. Confirm actual areas, extensions, revisions and generation rules before final design.

# 5. Source serviceability and utilisation audit

| Bridge | Vertical / limit mm | Ratio | Lateral / limit mm | Ratio | Listed member / joint UC |
| --- | --- | --- | --- | --- | --- |
| ICW-ICG LD | 105/140 | 75.0% | 87/112 | 77.7% | 0.557/0.509 |
| ICP-ICG LD | 177/210 | 84.3% | 123/168 | 73.2% | 0.61/0.625 |
| BHS-SLQ LD | 32/105 | 30.5% | 76/84 | 90.5% | 0.916/0.859 |

The limits are independently recalculated from the criteria; deflections and UC values are source results, not results of a new model. Highest listed member/joint values are not independently verified full envelopes. No adequacy statement is made for changed loads or geometry. Do not infer allowable load increases from these ratios.

# 6. Bearing movements, thermal sensitivity and camber

Use directional differential platform movements and compatible rotations. Criteria translation demand is150% of the worst-case sum of absolute adjacent-platform translations. Hinged axial connection demand is150% of expected axial thrust. Bearing geometry, lateral/vertical guides, friction, uplift, travel, maintenance and tolerances require a checked support model. The location-tolerance clause is not an instruction to specify1m bearing travel.

Thermal expansion sensitivity: deltaL=alpha x L x deltaT; alpha=12x10^-6/C.10C and20C below are illustrative increments, not adopted site temperature ranges. Combine thermal and imposed movement compatibly; avoid counting the same movement twice.

| Bridge | mm per C | At10C mm | At20C mm |
| --- | --- | --- | --- |
| ICW-ICG LD | 0.672 | 6.72 | 13.44 |
| ICP-ICG LD | 1.008 | 10.08 | 20.16 |
| BHS-SLQ LD | 0.504 | 5.04 | 10.08 |

Camber must compensate dead-load deflection under the approved geometry and construction-setting assumptions. Source DL deflections are85,143 and32mm respectively. These values are references, not adopted fabrication camber. The source-study1:200 statement requires reconciliation; no L/200 camber is issued.

# 7. Source section geometry

The ICP VIV table lists CHS406x15.9,406x12.7,168x9.5 and114x7.9mm. Workbook calculates ideal uncorroded A,I,Z,r and mass. It does not assign these to chords/braces, select sections, establish effective lengths or calculate code resistance.

| OD x t mm | Area mm2 | I mm4 | Mass kg/m |
| --- | --- | --- | --- |
| 406x15.9 | 19486.0 | 3.713e+08 | 152.97 |
| 406x12.7 | 15692.0 | 3.037e+08 | 123.18 |
| 168x9.5 | 4730.5 | 1.491e+07 | 37.13 |
| 114x7.9 | 2633.3 | 3.726e+06 | 20.67 |

# 8. Addendum jacket wrap steel: preliminary quantity calculation

For a close-fitting external wrap idealised as a full annulus: V=pi/4[(D+2t)^2-D^2]L, with consistent units; mass=Vx7850. Four-quarter fabrication, fit-up gaps, existing corrosion, welds, ends, allowances and local attachments require detailing. This is only geometric steel accounting, not strengthening-capacity verification or a purchase quantity.

| Platform | Existing OD x wall mm | Wrap length m | Wrap t mm | Ideal shell mass MT |
| --- | --- | --- | --- | --- |
| ICG | 660x12.7 | 15 | 10 | 2.478 |
| ICP | 762x16 | 13.5 | 12 | 3.092 |
| BHS | 457x12.7 | 3 | 10 | 0.346 |

# 9. Source-based detailing approach

Nine P4 development sheets cover three source-informed arrangements, three support/interface schematics and three bridge-specific fabrication/detailing schedules. Six original landing sheets are appended unchanged as tender-source references. New IDs are Shivansh development IDs and are not substitutions for EIL drawing numbers.

Dimensions and source sections are not inferred from screenshot scale. Connections, welds, bolt sizes, bearing sizes, guide gaps, lift points, member thicknesses and final camber are left explicitly unselected pending corresponding calculations. This preserves a usable preliminary package without fabricating detailed-design evidence.

# 10. Model reconstruction and engineering continuation

Verify actual nodes, axes, member connectivity, offsets, end releases, section groups and support freedoms.

Reproduce source gravity loadcases first and reconcile applied load totals, reactions and deflections.

Apply confirmed bridge-specific wind duration, environmental adjustment, service loads and allowances.

Check members for applicable strength, buckling, slenderness, fatigue and serviceability; check joints locally.

Transfer concurrent signed reaction envelopes to landing/deck/jacket models and incorporate addendum strengthening.

Develop operation-specific loadout, transport, lifting and temporary-condition analyses.

Issue preliminary framing/connection/bearing schedules with calculation references; resolve review comments before any later construction release.

# 11. Open-item register

Q01 | Final bearing centres, levels, skew and condition | OPEN | Survey-dependent geometry; model length is not shop length

Q02 | Native analysis connectivity/member groups/load map | OPEN | No independently reproduced FE model in this package

Q03 | Actual live-load areas and report/model differences | OPEN | Rectangle screens do not replace documented load generation

Q04 | Camber instruction in reports versus criteria rule | PARTLY RESOLVED | Dead-load compensation governs; profile and construction datum still required

Q05 | BHS SLQ support wording | OPEN | Report and methodology support fixed intent; SOW conflicting wording needs formal resolution

Q06 | Wind duration difference | EXPLAINED | 15sec for>50m;3sec for<50m; height/speed derivation pending

Q07 | Source member schedule / code editions / effective lengths | OPEN | VIV list is not a full member schedule

Q08 | Signed concurrent support reactions / existing capacity | OPEN | Summary values cannot be assembled into a fictitious concurrent case

Q09 | Missing101/102 main GA and0005;103series drawings | OPEN | Existing drawings and source studies used; gaps remain recorded

Q10 | Material grade, toughness and corrosion application | OPEN | 50ksi yield in study does not establish full material procurement specification

Q11 | BHS piping20%future allowance | OPEN | Confirm whether source1440kN already contains it; avoid duplication

Q12 | Weights and lift modules | OPEN | Tender MT values are not verified lifted masses

Q13 | MTO1MT difference | OPEN | Existing landing strengthening rows216vs217MT summary

Q14 | Other-project documents | CONTROLLED | BRRP PartII adopted; S&SRP/WSSRP-2 project-specific scope excluded unless directed

# 12. Review status

Completed: criteria consolidation, source/assumption traceability, load reconciliation arithmetic, source serviceability arithmetic, thermal sensitivities, source section geometry, ideal addendum wrap quantities, source-based drawing sheets and input/review registers. Outstanding: independent global model and resistance design, survey verification, full member schedule, reaction envelopes, permanent and temporary connection/bearing/lifting calculations. Construction release remains deferred.